SECTION 013513.13 - SITE SECURITY AND HEALTH REQUIREMENTS (DESE)

GENERAL

RELATED DOCUMENTS

Drawings and general provisions of the Contract including General and Supplementary Conditions, Bid Form, and other Division 1 Specification Sections apply to this Section.

SUMMARY

This Section includes general Institution rules.

This Section includes requirements for environments that employees are domiciled in, or public participation in program activities in or adjacent to the Scope of Work area:

The Contractor shall have the applicable measures specified below in-place any time demolition or construction activities occur in occupied or non-occupied project work areas.

The Contractor shall complete all specified cleaning procedures and receive clearance from the Construction Representative prior to removing any barriers and other precautionary measures – even for areas that the employees or public do not occupy during construction.

SUMMARY

This Section includes general Institution rules.

This Section includes requirements for environments that employees are domiciled in, or public participate in program activities in or adjacent to the Scope of Work area:

The Contractor shall have the applicable measures specified below in-place any time demolition or construction activities occur in occupied or non-occupied project work areas.

- The Contractor shall complete all specified cleaning procedures and receive clearance from the Construction Representative prior to removing any barriers and other precautionary measures – even for areas that the employees or public do not occupy during construction.

SUBMITTALS

List of required submittals:

Materials Safety Data Sheets for all hazardous materials to be brought onsite.

Schedule of proposed shutdowns, if applicable.

A list of the names of all employees who will submit fingerprints for a background check, and the signed privacy documents identified below for each employee.

“No Asbestos and No Lead” certification.

Drug testing program and certification.

PRODUCTS (Not Applicable)

EXECUTION

ACCESS TO THE SITE

The Contractor shall arrange with Facility Representatives to establish procedures for the controlled entry of workers and materials into the work areas at the Facility.

The Contractor shall establish regular working hours with Facility Representatives. The Contractor must report changes in working hours or overtime to Facility Representatives and obtain approval twenty-four (24) hours ahead of time. The Contractor shall report emergency overtime to Facility Representatives as soon as it is evident that overtime is needed. The Contractor must obtain approval from Facility Representatives for all work performed after dark.

The Contractor shall provide the name and phone number of the Contractor’s employee or agent who is in charge onsite; this individual must be able to be contacted in case of emergency. The Contractor must be able to furnish names and address of all employees upon request.

All construction personnel shall visibly display issued identification badges.

RULES OF THE FACILITY

No alcohol, drugs, guns, or other weapons are permitted anywhere at the Facility (i.e., inside or outside buildings, or anywhere on school grounds); violators will be referred to local law enforcement for prosecution.

No tobacco or smoking products may be used anywhere at the Facility.

Sexual harassment, offensive or fraternizing behavior, or foul language around or towards students or staff will not be tolerated. Violations by workers will result in one warning from the Facility Representative. Subsequent infractions will require permanent ejection of offending worker(s) from the jobsite, with no change to the contract schedule or additional cost to the State.

The Contractor shall consider the safety of the Facility’s students at all times, and shall maintain excavations, scaffolding/ladders, equipment, tools, and materials in as safe a manner as possible during and after working hours.

Vehicles should be locked and parked in areas designated by the Facility Representative.

Neither the Owner nor DESE assumes responsibility for the Contractor’s vehicles, equipment, tools, or materials.

The Contractor shall coordinate and communicate planned daily work activities with the Facility Representative at least two (2) working days in advance. This will allow time for the Facility Representative to consider temporarily relocating special education students whose health could be adversely affected by loud noises, chemical odors, temperature extremes, etc.

SECURITY CLEARANCES AND RESTRICTIONS

FMDC CONTRACTOR BACKGROUND AND ID BADGE PROCESS

All employees of the Contractor are required to submit fingerprints to the Missouri State Highway Patrol to enable the Office of Administration, Division of Facilities Management, Design and Construction (FMDC) to receive state and national criminal background checks on such employees. FMDC reserves the right to prohibit any employee of the Contractor from performing work in or on the premises of any facility owned, operated, or utilized by the State of Missouri for any reason.

The Contractor shall ensure all of its employees submit fingerprints to the Missouri State Highway Patrol and pay for the cost of such background checks. The Contractor shall submit to FMDC via email to FMDCContractorScreenings@oa.mo.gov a list of the names of the Contractor’s employees who will be fingerprinted and a signed Missouri Applicant Fingerprint Privacy Notice, Authorization For Release Of Information Confidentiality Oath and State Identification Badge Agreement for each employee. All employees of the Contractor approved by FMDC to work at a State facility must obtain a contractor ID badge from FMDC prior to beginning work on-site, unless the Director of FMDC, at the Director’s discretion, waives the requirement for a contractor ID badge. The Contractor and its employees must comply with the process for background checks and contractor ID badges found on FMDC’s website at: https://oa.mo.gov/fmdc-contractor-id-badges.

Pursuant to section 43.540, RSMo, FMDC participates in the Missouri Rap Back and National Rap Back programs as of August 28, 2018. This means that the Missouri State Highway Patrol, Central Records Repository, and the Federal Bureau of Investigation will retain the fingerprints submitted by each of the Contractor’s employees, and those fingerprints will be searched against other fingerprints on file, including latent fingerprints. While retained, an employee’s fingerprints may continue to be compared against other fingerprints submitted or retained by the Federal Bureau of Investigation, including latent fingerprints.

As part of the Missouri and National Rap Back programs, FMDC will receive notification if a new arrest is reported for an employee whose fingerprints have been submitted for FMDC after August 28, 2018. If the employee is performing work on a State contract at the time of the arrest notification, FMDC will request and receive the employee’s updated criminal history records. If the employee is no longer performing work on a State contract, FMDC will not obtain updated criminal records.

Pursuant to section 43.540, RSMo, the Missouri State Highway Patrol will provide the results of the employee’s background check directly to FMDC. FMDC may NOT release the results of a background check to the Contractor or provide the Contractor any information obtained from a background check, either verbally or in writing. FMDC will notify the Contractor only whether an employee is approved to work on State property.

Each employee who submits fingerprints to the Missouri State Highway Patrol has a right to obtain a copy of the results of his or her background check. The employee may challenge the accuracy and completeness of the information contained in a background check report and obtain a determination from the Missouri State Highway Patrol and/or the FBI regarding the validity of such challenge prior to FMDC making a final decision about his or her eligibility to perform work under a State contract.

The Contractor shall notify FMDC via email to FMDCContractorScreenings@oa.mo.gov if an employee is terminated or resigns from employment with the Contractor. If the Contractor does not anticipate performing work on a State contract in the future, the Contractor may request that FMDC remove its employees from the Rap Back programs. However, if removed from the Rap Back programs, employees will be required to submit new fingerprints should the contractor be awarded another State contract.

Upon award of a Contract, the Contractor should contact FMDC at FMDCContractorScreenings@oa.mo.gov to determine if its employees need to provide a new background check. If a Contractor’s employee has previously submitted a fingerprint background check to FMDC as part of the Missouri and National Rap Back programs, the employee may not need to submit another fingerprint search for a period of three to six years, depending upon the circumstances. The Contractor understands and agrees that FMDC may require more frequent background checks without providing any explanation to the Contractor. The fact that an additional background check is requested by FMDC does not indicate that the employee has a criminal record.

FIRE PROTECTION, SAFETY, AND HEALTH CONTROLS

The Contractor shall take all necessary precautions to guard against and eliminate possible fire hazards.

Onsite burning is prohibited.

The Contractor shall store all flammable or hazardous materials in proper containers located outside the buildings or offsite, if possible.

The Contractor shall provide and maintain, in good order, during construction fire extinguishers as required by the National Fire Protection Association. In areas of flammable liquids, asphalt, or electrical hazards, 15-pound carbon dioxide or 20-pound dry chemical extinguishers shall be provided.

The Contractor shall not obstruct streets or walks without permission from the Owner’s Construction Representative and Facility Representatives.

The Contractor’s personnel shall not exceed the speed limit of 15 mph while at the Facility unless otherwise posted.

The Contractor shall take all necessary, reasonable measures to reduce air and water pollution by any material or equipment used during construction. The Contractor shall keep volatile wastes in covered containers and shall not dispose of volatile wastes or oils in storm or sanitary drains.

The Contractor shall keep the project site neat, orderly, and in a safe condition at all times. The Contractor shall immediately remove all hazardous waste and shall not allow rubbish to accumulate. The Contractor shall provide onsite containers for collection of rubbish and shall dispose of it at frequent intervals during the progress of the Work.

Fire exits, alarm systems, and sprinkler systems shall remain fully operational at all times, unless written approval is received from the Owner’s Construction Representative and the appropriate Facility Representative at least twenty-four (24) hours in advance. The Contractor shall submit a written time schedule for any proposed shutdowns.

For all hazardous materials brought onsite, Material Safety Data Sheets shall be on site and readily available upon request at least a day before delivery.

The Contractor’s workers shall not be under the influence of any intoxicating substances while on the Facility premises.

NO ASBESTOS AND NO LEAD CERTIFICATION REQUIREMENTS

No asbestos containing material (1.0% asbestos by dry weight) or lead containing material ( 0.06% or 600ppm/10,000ppm lead by dry weight) shall be included in any project submittals or physically installed during construction work on this project.

USEPA regulations exclude local education agencies (i.e., DESE MSB, MSD, & SSSH) from the requirements of inspection, sampling, and analysis of homogenous areas that have been newly constructed or repaired/replaced in special education school buildings; where an Architect or Project Engineer responsible for the construction, or an Accredited Inspector, provides a signed statement that no asbestos (or lead) was specified, or used, as a building material (or system component) in any project construction documents, or physically installed as part of the project work. It is recommended that the Contractor research each material/component used on the job to verify that it contains no asbestos or lead (i.e., look at manufacturer’s cut-sheet specifications, Material Safety Data Sheets, DOT shipping classification, or even contact the manufacturer for their verification); then, the Contractor should write on each project submittal: “To the best of my knowledge, items covered by this submittal contain no asbestos or lead containing material”.

Contractor Certification Requirement

Prior to final payment, the Contractor shall submit a signed letter on company letterhead certifying that, to the best of its knowledge, no asbestos or lead containing materials were used or installed during the work. The Contractor shall address the letter to the Service Level Manager/ Designated Person for FMDC, at P.O. Box 809, Jefferson City, MO 65102, and (if applicable) to the Architect or Project Engineer. The letter shall reference the Site/Facility Name, Project Number, Project Title, and shall include the following statement:

“The Contractor certifies, to the best of its knowledge, that no asbestos containing material (1.0% asbestos by dry weight) or lead containing material (0.06% or 600ppm/10,000ppm lead by dry weight) was included in any project submittals or physically installed during construction work on this project. Contractor agrees to pay all costs incurred by the Owner discovering, abating, and/or restoring any component or portion of the work that is later found to include an asbestos or lead containing material in excess of these limitations.”

Architect or Project Engineer Certification

As part of the final as built/close-out document submittal requirements, it is requested that the Project Architect or Engineer (or Accredited Inspector as a last resort) responsible for design and submittal approval, submit a signed letter on company letterhead that references the Site/Facility Name, Project Number, Project Title, and includes the following statement:

“As the Designer, or Accredited Inspector, I certify, to the best of my knowledge, that no asbestos containing material (1.0% asbestos by dry weight) or lead containing material (0.06%, or, 600ppm/10,000ppm lead by dry weight) was specified in the construction documents, or approved for installation by the Contractor during construction work, on this project.”

DRUG & ALCOHOL TESTING PROGRAM CONTRACTUAL REQUIREMENT (1 CSR 30-7.010)

BASIS AND LEGAL REQUIREMENTS

In an effort to create safe and healthy schools and workplaces, the State of Missouri requires that Contractors and Subcontractors shall maintain and enforce a written substance abuse testing program for public works construction projects on public and charter elementary and secondary education construction projects that are subject to the control of the State of Missouri. This policy is not intended to be a substitute for the Contractor’s or Subcontractor’s complete written substance abuse policy. These requirements shall be the minimum requirements for complying with Section 161.371, RSMo, and may be supplemented at the discretion of the Contractor or Subcontractor.

The State of Missouri has a vital interest in protecting the safety of students and maintaining safe, healthful, and efficient working conditions for both the state and its’ Contractors’ and Subcontractors’ employees; and has determined that the educational and work environment is safer and more productive without the presence of illegal or inappropriate drugs, alcohol, or other substances in the body or on state property on which any state elementary or secondary school is located or being constructed or improved.

The use of illegal drugs, on or off duty, is inconsistent with law-abiding behavior expected of all persons. The use of illegal drugs, or abuse of alcohol or prescription drugs, may impair the ability of employees to perform tasks that are critical to proper work performance. The result is an increase in accidents and failures that pose a serious threat to the safety of all students, employees, visitors and the general public. Impaired employees also tend to be less productive, less reliable and prone to greater absenteeism, resulting in the potential for increased cost and delays in the timely completion of contracts.

CONTRACTUAL REQUIREMENTS

Each contract entered into for the performance of work on any public and charter elementary or secondary project subject to the control of the State of Missouri shall require that each Contractor and each Subcontractor have in place a drug and alcohol testing program consistent with this rule. These contractual requirements shall apply to Contractor and Subcontractor employees on public and charter elementary and secondary education construction projects that are subject to the control of the State of Missouri, including workers, new hires, replacements, and supervisory personnel. The Contractor and all Subcontractors shall comply with this contractual requirement. The State of Missouri shall determine, in its sole discretion, when this contractual requirement shall be applicable; and in such instances, any bid submitted in response to a request for proposal shall comply with this contractual requirement.

In order to be eligible to perform work on public and charter elementary and secondary education construction projects that are subject to the control by the State of Missouri, a Contractor must have and enforce a written drug and alcohol testing program incorporating the following testing requirements, terms and conditions applicable to all its employees, prospective employees and Subcontractors. Neither employee nor prospective employee of a Contractor or Subcontractor shall be permitted to work on public and charter elementary and secondary education construction projects that are subject to this rule unless such employee submits to testing as required by the contractual requirement required by this rule.

Each Contractor and Subcontractor subject to this rule shall train its’ supervisory employees in methods that will allow them to recognize the signs and symptoms of substance abuse and to take action provided by this contractual requirement in a manner consistent with generally accepted safety training procedures.

Each Contractor and Subcontractor subject to this rule is responsible for the cost of developing, implementing, and enforcing its drug and alcohol testing program, including the cost of drug and alcohol testing of its employees provided by the contractual requirement required by this rule.

Each Contractor shall furnish a copy of its drug and alcohol testing program and certify that it and its’ Subcontractors are in compliance with the provisions of this rule to the State of Missouri at the time it submits a bid for any contract with the State of Missouri for work on public and charter elementary and secondary education construction projects that are subject to the control of the State of Missouri. Additionally, each Subcontractor shall furnish a copy of its substance abuse testing program to the Contractor prior to commencement of work on public and charter elementary and secondary education construction projects that are subject to this contractual requirement. The Contractor may reject a Subcontractor’s program as noncompliant with the contractual requirement required by this rule.

TESTING REQUIREMENTS

PRE-ENGAGEMENT TESTING: Testing for all substances other than alcohol as described in this rule shall be conducted by each Contractor and Subcontractor for its employees or prospective employees within 120 days prior to any employee’s appearance on a public and charter elementary and secondary education construction project that is subject to this contractual requirement. Contractors’ or Subcontractors’ employees that can provide certification of a previous drug test occurring within 120 days or employees that have been subject during the preceding consecutive two (2) years to a random and periodic selection program that meets the standards as set forth in this rule and, if the employee actually has been tested, that indicates a negative result for each of the substances listed herein, may be exempted from pre-engagement testing provided by this rule. If the employee was not employed by the Contractor or Subcontractor that is his or her current employer at the time of the previous test, the employee may be exempted from pre-engagement testing only upon certification of the non-negative test directly from the administrator of the testing program that conducted the previous test.

RANDOM TESTING: All employees of the Contractor and Subcontractor shall be subject to random testing by the Contractor or Subcontractor. For employees holding a commercial driver license, the annualized drug and alcohol testing rate shall comply with 49 CFR Part 382, as may be amended from time to time and similar applicable regulations of the Federal Highway Administration. All other employees of the Contractor or Subcontractor shall be subject to testing for all substances other than alcohol at the random annualized selection rate of fifty (50) percent of the Contractor’s or Subcontractor’s employees. Employees selected for random testing shall report in a timely manner to the drug and alcohol testing laboratory or collection site where directed for drug and/or alcohol testing.

PERIODIC TESTING: All employees working on public and charter elementary and secondary education construction projects that are subject to this rule shall be subject to periodic and random testing for all substances other than alcohol on at least a biannual basis. Employees subject to periodic testing shall report in a timely manner as directed to the drug and alcohol testing laboratory or collection site for drug testing.

REASONABLE SUSPICION TESTING: All employees of the Contractor and Subcontractor on public and charter elementary and secondary education construction projects that are subject to this rule shall be subject to a drug and alcohol test when an employee is acting in an abnormal manner that leads a supervisory employee of the Contractor or Subcontractor to have reasonable suspicion that the employee is under the influence of alcohol or controlled substances. Reasonable suspicion means suspicion based on specific personal observations by the supervisory employee concerning the appearance, behavior, speech or breath odor of the employee.

POST-ACCIDENT/INCIDENT TESTING: All employees of Contractors and Subcontractors on public and charter elementary and secondary education construction projects who are subject to this rule shall be subject to a drug and alcohol test following an on-the-job injury requiring medical treatment or following a serious or potentially serious incident, including near misses, during which safety precautions were violated, persons were or could have been injured, unsafe instructions or orders were given, vehicles, equipment, or property was damaged, careless acts were performed, or when prescribed personal protective or safety equipment was not worn. Employees involved or who may have contributed to the incident, shall be subject to a drug and alcohol test. If it is impossible or impractical, because of the physical condition of the person involved in the accident to be subjected to drug and alcohol testing; and if in subsequent medical treatment, that person’s blood or other bodily fluid will be drawn, then that blood or other bodily fluids may be analyzed for drugs and alcohol.

SUBSTANCE ABUSE TESTING PROTOCOLS

A Contractor or Subcontractor subject to the provisions of this rule shall perform pre-engagement, random, periodic, reasonable suspicion, and post-accident/incident testing in the following manner:

Drug Testing

All urine samples collected under this program shall be analyzed by a laboratory certified by the National Institute on Drug Abuse/Substance Abuse and Mental Health Service Administration of the U.S. Department of Health and Human Services and shall include an initial Enzyme Multiplied Immunoassay Screening Test (EMIT) and, when necessary, confirmed by a Gas Chromatography /Mass Spectrometry (GC/MS) confirmation test. All samples confirmed by the laboratory as non-negative shall be interpreted as positive or negative by a Medical Review Officer licensed by the American Association of Medical Review Officers, American College of Occupational and Environmental Medicine, Medical Review Officer Certification Council, or American Society of Addiction Medicine.

Alcohol Testing

The initial screening tests for alcohol shall be performed by using either a saliva test or a DOT approved breathalyzer.

Alcohol confirmatory tests shall be performed by either a blood alcohol test or a DOT approved breathalyzer.

Testing for the presence of drugs or alcohol in an employee's system and the handling of test specimens shall be conducted in accordance with guidelines for laboratory testing procedures and chain-of-custody procedures established by the Substance Abuse and Mental Health Service Administration of the U.S. Department of Health and Human Services.

The program shall require notification to the employer and employee of the results of any non-negative drug and alcohol test and the Division of Facilities Management, Design and Construction shall be notified of the action taken to protect the safety of students as a result of such positive test, provided that no requirement of individual confidentiality of test results provided by federal law or regulation or state statute shall be violated in providing such notifications.

THRESHOLD LIMITS

All samples collected shall be analyzed by a laboratory certified by the Substance Abuse and Mental Health Service Administration of the U.S. Department of Health and Human Services, and shall include an initial Enzyme Multiplied Immunoassay Screening Test (EMIT) and, when necessary, confirmed by a Gas Chromatography/Mass Spectrometry (GC/MS) Confirmation Test. Said testing must screen, at a minimum, for the substances and levels of such substances provided by 49 CFR Part 40 and for alcohol as provided by 49 CFR Part 382, as may be amended from time to time. The levels that shall be deemed to result in a negative test result shall be defined by 49 CFR Part 40 and 49 CFR Part 382, as may be amended from time to time; provided that if such regulations shall no longer define substances and testing levels in the future, testing as required by this rule shall screen for the following substances that shall not exceed the following levels in order to be deemed a negative test result:

(EMIT) CONFIRMED/(GC/MS) CONFIRMATION TEST:

Drug tested/ Initial Level(ng/ml)/ Cut-Off Level(ng/ml)

Amphetamines/500/250 - Includes Amphetamines, Methamphetamines and Ecstasy (MDMA)

Barbiturates/300/200

Benzodiazepines/300/200

Cocaine Metabolite/150/100

Cannabinoids (Marijuana THC)/50/15

Methadone/300/200

Opiates:

Codeine/Morphine/2000/2000

Heroine Metabolite/10/10

Phencyclidine (PCP)/25/25

Propoxyphene/300/200

Breath/Blood Alcohol Content (BAC)/.04%/.04%

Removal from jobsite (BAC)/.0200-.0399%/.0200%-.0399%

REFUSAL TO SUBMIT TO TESTING/CONFIRMED POSITIVE RESULTS

Any employee of a Contractor or Subcontractor performing any duties or work that are subject to this rule who refuses to submit to testing or receives a confirmed positive test result for any of the substances indicated in Section E shall be required to immediately leave the construction site and be prohibited from returning to any construction site subject to control of the State of Missouri until evidence is provided of the completion of the reinstatement procedures as set forth in section G.

Determination for Violation of Policy

A confirmed positive drug or alcohol test.

Failure to contact the Medical Review Officer as directed.

Failure to report as directed for random testing.

The use, possession, sale or distribution of alcohol or a controlled illegal or unauthorized substance, or the presence of any employee with such ingested substances for non-medical reasons on a public and charter elementary and secondary education construction project subject to the control of the State of Missouri.

Working, reporting to work, being on a public and charter elementary and secondary education construction project that is subject to the control of the State of Missouri, or in a state or employer owned, leased or rented vehicle, while under the influence of alcohol (0.04% BAC or greater).

Switching, adulterating or attempting to tamper with any sample submitted for drug or alcohol testing or otherwise interfering or attempting to interfere with the testing process.

Refusal to submit a specimen for testing shall be deemed to be a positive test result and shall be subject to the same consequences as specimens tested and confirmed as positive.

The use of a controlled substance by an individual other than the individual for whom the controlled substance was prescribed or the abuse of a controlled substance by the individual for whom it was prescribed.

REINSTATEMENT PROCEDURES

An employee receiving a confirmed positive test result for any of the substances indicated in Section 5 may return to work on a public and charter elementary and secondary education construction project that is subject to the control of the State of Missouri only after the following conditions have been satisfied:

Evidence is submitted to the Contractor or Subcontractor that the employee has completed or is actively participating in an approved drug/alcohol assessment, treatment, and/or counseling program. The costs of this assessment, treatment or program need not be borne by the Contractor or Subcontractor.

Evidence is submitted of the employee passing of a drug and alcohol test that meets the requirements of Sections E and F of this rule. The costs of this subsequent retesting need not be borne by the Contractor or Subcontractor.

The employee shall be subject to additional random drug and alcohol testing on a monthly basis while on any public and charter elementary and secondary education construction project that is subject to the control of the State of Missouri. The costs of this additional testing, treatment or program need not be borne by the Contractor or Subcontractor.

An employee known by the Contractor or Subcontractor to have previously had a positive test result who receives a second or subsequent confirmed positive test result in connection with subsequent testing required by this Section H of this rule shall be removed by the Contractor or Subcontractor from all public and charter elementary and secondary education construction projects that are subject to the control of the State of Missouri. The employee shall not return to work on any public and charter elementary and secondary education construction project subject to this rule until that the employee has completed an approved drug/alcohol assessment, treatment, and/or counseling program; and until after evidence is submitted of the employee passing of a drug and alcohol test that meets the requirements of sections E and F of this rule and that indicates a blood alcohol concentration of less than 0.02 percent.

COMPLIANCE DETERMINATION

The State of Missouri may audit any substance abuse testing program implemented pursuant to this contractual requirement to verify compliance, upon at least 24 hours notice by the State to the Contractor of its intent to audit. The State shall have free access to all relevant records of the Contractor and its Subcontractors for this purpose.

Any portion of this program that is in violation of applicable federal or state law or regulation shall be deemed unenforceable.

DISRUPTION OF UTILITIES

The Contractor shall give a minimum of 72 hours written notice to the Construction Representative and Facility Representative before disconnecting electric, gas, water, fire protection, or sewer service to any building.

The contractor shall give a minimum of 72 hours written notice to the Construction Representative and Facility Representative before closing any access drives and shall make temporary access available if possible. The contractor shall not obstruct streets, walks, or parking.

PROTECTION OF PERSONS AND PROPERTY

SAFETY PRECAUTIONS AND PROGRAMS

The Contractor shall at all times conduct operations under this Contract in a manner to avoid the risk of bodily harm to persons or risk of damage to any property. The Contractor shall promptly take precautions which are necessary and adequate against conditions created during the progress of the Contractor's activities hereunder which involve a risk of bodily harm to persons or a risk of damage to property. The Contractor shall continuously inspect Work, materials, and equipment to discover and determine any such conditions and shall be solely responsible for discovery, determination, and correction of any such conditions. The Contractor shall comply with applicable safety laws, standards, codes, and regulations in the jurisdiction where the Work is being performed, specifically, but without limiting the generality of the foregoing, with rules regulations, and standards adopted pursuant to the Williams-Steiger Occupational Safety and Health Act of 1970 and applicable amendments.

All contractors, subcontractors and workers on this project are subject to the Construction Safety Training provisions 292.675 RSMo.

In the event the Contractor encounters on the site, material reasonably believed to be asbestos, polychlorinated biphenyl (PCB), lead, mercury, or other material known to be hazardous, which has not been rendered harmless, the Contractor shall immediately stop Work in the area affected and report the condition to the Owner's Representative and the Architect in writing. The Work in the affected area shall not thereafter be resumed except by written agreement of the Owner's Representative and Contractor if in fact the material is asbestos or polychlorinated biphenyl (PCB) and has not been rendered harmless. The Work in the affected area shall be resumed in the absence of asbestos or polychlorinated biphenyl (PCB), or when it has been rendered harmless by written agreement of the Owner's Representative and the Contractor. “Rendered Harmless” shall mean that levels of such materials are less than any applicable exposure standards, including but limited to OSHA regulations.

SAFETY OF PERSONS AND PROPERTY

The Contractor shall take reasonable precautions for safety of, and shall provide protection to prevent damage, injury, or loss to:

clients, staff, the public, construction personnel, and other persons who may be affected thereby;

the Work and materials and equipment to be incorporated therein, whether in storage on or off the site, under care, custody, or control of the Contractor or the Contractor's Subcontractors of any tier; and

other property at the site or adjacent thereto, such as trees, shrubs, lawns, walks, pavements, roadways, structures, and utilities not designated for removal, relocation, or replacement in the course of construction.

The Contractor shall give notices and comply with applicable laws, standards, codes, ordinances, rules, regulations, and lawful orders of public authorities bearing on safety of persons or property or their protection from damage, injury, or loss.

The Contractor shall erect and maintain, as required by existing conditions and performance of the Contract, safeguards for safety and protection, including, but not limited to, posting danger signs and other warnings against hazards, promulgating safety regulations, and notifying owners and users of adjacent sites and utilities.

When use or storage of explosives or other hazardous materials or equipment or unusual methods are necessary for execution of the Work, the Contractor shall exercise the highest degree of care and carry on such activities under supervision of properly qualified personnel.

The Contractor shall promptly remedy damage and loss (other than damage or loss insured under property insurance required by the Contract Documents) to property referred to in this Section caused in whole or in part by the Contractor, a Subcontractor of any tier, or anyone directly or indirectly employed by any of them, or by anyone for whose acts they may be liable, and for which the Contractor is responsible under this Section, except damage or loss attributable solely to acts or omissions of Owner or the Architect or anyone directly or indirectly employed by either of them, or by anyone for whose acts either of them may be liable, and not attributable to the fault or negligence of the Contractor. The foregoing obligations of the Contractor are in addition to the Contractor's other obligations stated elsewhere in the Contract.

The Contractor shall designate a responsible member of the Contractor's organization at the site whose duty shall be the prevention of accidents, and the maintaining, enforcing and supervising of safety precautions and programs. This person shall be the Contractor's superintendent unless otherwise designated by the Contractor in writing to the Owner's Representative and Architect. The Contractor shall hold regularly scheduled safety meetings to instruct Contractor personnel on safety practices, accident avoidance and prevention, and the Project Safety Program. The Contractor shall furnish safety equipment and enforce the use of such equipment by its employees and its subcontractors of any tier.

The Contractor shall not load or permit any part of the construction or site to be loaded so as to endanger its safety.

The Contractor shall promptly report in writing to the Owner all accidents arising out of or in connection with the Work which cause death, lost time injury, personal injury, or property damage, giving full details and statements of any witnesses. In addition, if death, serious personal injuries, or serious property damages are caused, the accident shall be reported immediately.

The Contractor shall promptly notify in writing to the Owner of any claims for injury or damage to personal property related to the work, either by or against the Contractor.

The Owner assumes no responsibility or liability for the physical condition or safety of the Work site or any improvements located on the Work site. The Contractor shall be solely responsible for providing a safe place for the performance of the Work. The Owner shall not be required to make any adjustment in either the Contract Sum or Contract Time concerning any failure by the Contractor or any Subcontractor to comply with the requirements of this Paragraph.

In no event shall the Owner have control over, charge of, or any responsibility for construction means, methods, techniques, sequences or procedures or for safety precautions and programs in connection with the Work, notwithstanding any of the rights and authority granted the Owner in the Contract Documents.

The Contractor shall maintain at his own cost and expense, adequate, safe and sufficient walkways, platforms, scaffolds, ladders, hoists and all necessary, proper, and adequate equipment, apparatus, and appliances useful in carrying on the Work and which are necessary to make the place of Work safe and free from avoidable danger for clients, staff, the public and construction personnel, and as may be required by safety provisions of applicable laws, ordinances, rules regulations and building and construction codes.

END OF SECTION 013513.13