Copyright 2005, The Missouri Division of Facilities Management, Design and Construction  (D&C)

SECTION 017400 – CLEANING

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions, Bid Form and other Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes administrative and procedural requirements for cleaning during the Project.

        - Environmental Requirements: Conduct cleaning and waste-disposal operations in compliance with local laws and ordinances.  Comply fully with federal and local environmental and anti-pollution regulations.

Subparagraphs below are examples of common environmental and anti-pollution requirements.  Delete if requirements are inappropriate. Insert other requirements to suit Project.

          - Do not dispose of volatile wastes, such as mineral spirits, oil, or paint thinner, in storm or sanitary drains.

          - Burning or burying of debris, rubbish, or other waste material on the premises is not permitted.

- PRODUCTS

      - MATERIALS

Paragraph below contains basic requirements.  Modify requirements to suit Project or to accommodate unusual situations.

        - Cleaning Agents:  Use cleaning materials and agents recommended by the manufacturer or fabricator for the surface to be cleaned.  Do not use cleaning agents that are potentially hazardous to health or property or that might damage finished surfaces.

- EXECUTION

      - PROGRESS CLEANING

Modify conditions and frequencies below to suit Project.

        - General

          - Retain all stored items in an orderly arrangement allowing maximum access, not impending drainage or traffic, and providing the required protection of materials.

          - Do not allow the accumulation of scrap, debris, waste material, and other items not required for construction of this Work.

          - At least <once><twice> each month, and more often if necessary, completely remove all scrap, debris, and waste material from the job site.

          - Provide adequate storage for all items awaiting removal from the job site, observing all requirements for fire protection and protection of the ecology.

Modify conditions and frequencies below to suit Project.

        - Site:

          - Daily, inspect the site and pick up all scrap, debris, and waste material.  Remove all such items to the place designated for their storage.

          - Weekly, inspect all arrangements of materials stored on the site.  Re-stack, tidy, or otherwise service all material arrangements.

          - Maintain the site in a neat and orderly condition at all times.

Modify conditions and frequencies below to suit Project.

        - Structures:

          - Daily, inspect the structures and pick up all scrap, debris, and waste material.  Remove all such items to the place designated for their storage.

          - Weekly, sweep all interior spaces clean. “Clean,” for the purposes of this paragraph, shall be interpreted as meaning free from dust and other material capable of being removed by use of reasonable effort and handheld broom.

          - In preparation for installation of succeeding materials, clean the structures or pertinent portions thereof to the degree of cleanliness recommended by the manufacturer of the succeeding material, using all equipment and materials required to achieve the required cleanliness.

          - Following the installation of finish floor materials, clean the finish floor daily while work is being performed in the space in which finish materials have been installed. “Clean,” for the purposes of this subparagraph, shall be interpreted as meaning free from all foreign material which, in the opinion of the Construction Representative, may be injurious to the finish of the finish floor material.

      - FINAL CLEANING

        - General:  Provide final cleaning operations when indicated.  Employ experienced workers or professional cleaners for final cleaning.  Clean each surface or unit of Work to the condition expected from a commercial building cleaning and maintenance program.  Comply with manufacturer’s instructions.

If final cleaning is delayed until final acceptance, modify paragraph below.

        - Complete the following cleaning operations before requesting inspection for certification of Substantial Completion for the entire Project or a portion of the Project.

Modify list below to suit Project.  Cover specific work, such as waxing floors or replacing filters, in individual sections.

          - Clean the Project Site, yard and grounds, in areas disturbed by construction activities, including landscape development areas, of rubbish, waste material, litter, and foreign substances.

          - Sweep paved areas broom clean.  Rake grounds that are neither planted nor paved to a smooth, even-textured surface.

          - Remove petrochemical spills, stains, and other foreign deposits.

          - Remove tools, construction equipment, machinery, and surplus material from the site.

          - Remove snow and ice to provide safe access to the building.

          - Clean exposed exterior and interior hard-surfaced finishes to a dirt-free condition, free of stains, films, and similar foreign substances.  Avoid disturbing natural weathering of exterior surfaces.  Restore reflective surfaces to their original condition.

          - Remove debris and surface dust from limited access spaces, including roofs, plenums, shafts, trenches, equipment vaults, manholes, attics, and similar spaces.

          - Broom clean concrete floors in unoccupied spaces.

          - Vacuum clean carpet and similar soft surfaces removing debris and excess nap.  Shampoo, if required.

          - Clean transparent material, including mirrors and glass in doors and windows.  Remove glazing compounds and other substances that are noticeable vision-obscuring materials. Replace chipped or broken glass and other damaged transparent materials.  Polish mirrors and glass, taking care not to scratch surfaces.

          - Remove labels that are not permanent labels.

          - Touch up and otherwise repair and restore marred, exposed finishes and surfaces.  Replace finishes and surfaces that cannot be satisfactorily repaired or restored or that already show evidence of repair or restoration.

            - Do not paint over “UL” and similar labels, including mechanical and electrical nameplates.

          - Wipe surfaces of mechanical and electrical equipment, elevator equipment, and similar equipment.  Remove excess lubrication, paint and mortar droppings, and other foreign substances.

          - Clean plumbing fixtures to a sanitary condition free of stains, including stains resulting from water exposure.

          - Replace disposable air filters and clean permanent air filters.  Clean exposed surfaces of diffusers, registers, and grills.

          - Clean ducts, blowers, and coils if units were operated without filters during construction

          - Clean, food-service equipment to a sanitary condition, ready and acceptable for its intended use.

          - Clean light fixtures, lamps, globes, and reflectors to function with full efficiency.  Replace burned-out bulbs and defective and noisy starters in fluorescent and mercury vapor fixtures.

          - Leave the Project clean and ready for occupancy.

        - Pest Control:  Engage an experienced, licensed exterminator to make a final inspection and rid the Project of rodents, insects, and other pests.  Comply with regulations of local authorities.

        - Removal of Protection: Remove temporary protection and facilities installed during construction to protect previously completed installations during the remainder of the construction period.

        - Compliances:  Comply with governing regulations and safety standards for cleaning operations.  Remove waste materials from the site and dispose of lawfully.

          - Where extra materials of value remain after Final Acceptance by the Owner, they become the Owner’s property.

Consider adding an article on continuing inspections or consultations by the Contractor.  Possibly insert a schedule of approximate times for inspections on principal-warranted items.

END OF SECTION 017400